Q8. Apply non-linear filters (median or other rank filters) to remove noise from data/images provided in OPE7 and perform comprehensive analysis.                                         				         	       CO3, CO4

Identify noise types present in the dataset (salt-and-pepper, impulse, speckle, etc.).Implement median filtering with varying window sizes and, optionally, other non-linear filters (alpha-trimmed mean, morphological filters).Analyze how filter size and type affect detail preservation vs. noise removal.
Observations: optimum filter sizes per noise type, edge preservation analysis, and failure cases.
Summarize best-performing methods for each noise type and practical recommendations.

Code:

!pip install -q scikit-image opencv-python matplotlib numpy scipy pandas

import cv2

import numpy as np

import matplotlib.pyplot as plt

from skimage import util, filters, morphology, exposure

from skimage.metrics import peak_signal_noise_ratio as PSNR, structural_similarity as SSIM

from scipy.ndimage import median_filter

import pandas as pd

from skimage.feature import canny

def show(title, img1, img2, l1="Original", l2="Filtered"):

plt.figure(figsize=(10,4))

plt.suptitle(title)

plt.subplot(1,2,1); plt.imshow(img1, cmap='gray'); plt.title(l1); plt.axis("off")

plt.subplot(1,2,2); plt.imshow(img2, cmap='gray'); plt.title(l2); plt.axis("off")

plt.show()

def to_float(img):

if img.dtype == np.uint8:

return img.astype(np.float32)/255.0

return img.astype(np.float32)

def to_uint8(img):

return np.clip(img*255.0,0,255).astype(np.uint8)

# Noise addition

def add_salt_pepper(img, amount=0.05):

return util.random_noise(img, mode='s&p', amount=amount)

def add_speckle(img, var=0.04):

return util.random_noise(img, mode='speckle', var=var)

def add_gaussian(img, var=0.004):

return util.random_noise(img, mode='gaussian', var=var)

def alpha_trimmed_mean(img, size=3, d=2):

pad = size//2

padded = np.pad(img, pad, mode='reflect')

out = np.zeros_like(img)

flat_n = size*size

trim = d//2

for i in range(out.shape[0]):

for j in range(out.shape[1]):

block = padded[i:i+size, j:j+size].flatten()

block_sorted = np.sort(block)

trimmed = block_sorted[trim:flat_n-trim]

out[i,j] = np.mean(trimmed)

return out

def edge_overlap(ref, proc):

ref_e = canny(ref)

proc_e = canny(proc)

inter = np.logical_and(ref_e, proc_e).sum()

return inter / max(ref_e.sum(),1)

image_paths = [

"/content/Picture1.png",

"/content/Picture2.png",

"/content/Picture3.png",

"/content/Picture4.png",

"/content/Picture5.png"

]

images = {}

for p in image_paths:

img = cv2.imread(p,0)

if img is not None:

images[p] = to_float(img)

print("Loaded:", p)

else:

print(" Missing:", p)

if not images:

raise RuntimeError("No images loaded.")

noisy_versions = {}

for p, img in images.items():

noisy_versions[p] = {

"salt_pepper": add_salt_pepper(img, amount=0.06),

"speckle": add_speckle(img, var=0.03),

"gaussian": add_gaussian(img, var=0.005)

}

print("Created noise versions for:", p)

median_sizes = [3,5,7,9]

alpha_sizes = [3,5]

alpha_d = 2

morph_sizes = [3,5,7]

rows = []

for p, noises in noisy_versions.items():

orig = images[p]

for noise_type, noisy in noises.items():

print(f"Image: {p} | Noise: {noise_type}")

# baseline

base_ps = PSNR(orig, noisy, data_range=1)

base_ss = SSIM(orig, noisy, data_range=1)

base_eo = edge_overlap(orig, noisy)

print("Baseline:", base_ps, base_ss, base_eo)

for k in median_sizes:

med = median_filter(noisy, size=k)

ps = PSNR(orig, med, data_range=1)

ss = SSIM(orig, med, data_range=1)

eo = edge_overlap(orig, med)

rows.append([p, noise_type, f"median_{k}", ps, ss, eo])

print(f"Median {k}: PSNR={ps:.2f} SSIM={ss:.4f} Edge={eo:.3f}")

for k in alpha_sizes:

at = alpha_trimmed_mean((noisy*255).astype(np.uint8), size=k, d=alpha_d)/255.0

ps = PSNR(orig, at, data_range=1)

ss = SSIM(orig, at, data_range=1)

eo = edge_overlap(orig, at)

rows.append([p, noise_type, f"alpha_trim_{k}", ps, ss, eo])

print(f"Alpha-trim {k}: PSNR={ps:.2f} SSIM={ss:.4f} Edge={eo:.3f}")

for s in morph_sizes:

se = morphology.square(s)

op = morphology.opening((noisy*255).astype(np.uint8), se)

cl = morphology.closing(op, se).astype(np.float32)/255.0

ps = PSNR(orig, cl, data_range=1)

ss = SSIM(orig, cl, data_range=1)

eo = edge_overlap(orig, cl)

rows.append([p, noise_type, f"morph_{s}", ps, ss, eo])

print(f"Morph {s}: PSNR={ps:.2f} SSIM={ss:.4f} Edge={eo:.3f}")

# Show best median visually

best = None

for k in median_sizes:

med = median_filter(noisy, size=k)

ss = SSIM(orig, med, data_range=1)

if best is None or ss > best[0]:

best = (ss, k, med)

show(f"{p} | {noise_type}: Best Median {best[1]} (SSIM={best[0]:.4f})",

to_uint8(orig), to_uint8(best[2]),

"Clean", "Best Median")

df = pd.DataFrame(rows, columns=["image","noise","method","psnr","ssim","edge_overlap"])

summary = df.sort_values(["image","noise","ssim","psnr"], ascending=[True,True,False,False])

print("Top performers:")

display(summary.head(25))

print("Practical Recommendations:")

print("1) Salt-and-pepper noise → Median 3x3 or 5x5, Morphological opening+closing")

print("2) Impulse noise → Median 5x5 or Alpha-trim filter (size 5)")

print("3) Speckle noise → Alpha-trim, median 7x7; morphological less effective")

print("4) Gaussian noise → Non-linear filters less effective; small median helps but may blur edges")

print("5) Edge preservation best → Median small window (3 or 5)")

print("6) Larger window sizes remove more noise but reduce fine detail.")

OUTPUT:

Baseline PSNR: 16.83, SSIM: 0.4157, EdgeOverlap: 0.6161

median 3: PSNR=27.32, SSIM=0.8325, EdgeOv=0.6396

median 5: PSNR=23.80, SSIM=0.6127, EdgeOv=0.2810

median 7: PSNR=22.63, SSIM=0.4818, EdgeOv=0.1347

median 9: PSNR=22.17, SSIM=0.4240, EdgeOv=0.0839

alpha-trim 3: PSNR=25.94, SSIM=0.7781, EdgeOv=0.6355

alpha-trim 5: PSNR=23.65, SSIM=0.5775, EdgeOv=0.2676

alpha-trim 7: PSNR=22.62, SSIM=0.4494, EdgeOv=0.0963

morph 3: PSNR=22.92, SSIM=0.6404, EdgeOv=0.3098

morph 5: PSNR=19.31, SSIM=0.3946, EdgeOv=0.0948

morph 7: PSNR=14.95, SSIM=0.2768, EdgeOv=0.0753

/tmp/ipython-input-4145038815.py:48: FutureWarning: `square` is deprecated since version 0.25 and will be removed in version 0.27. Use `skimage.morphology.footprint_rectangle` instead.

selem = morphology.square(s)

Best median size (by SSIM) = 3 (SSIM=0.8325)

Baseline PSNR: 21.54, SSIM: 0.6297, EdgeOverlap: 0.7454

median 3: PSNR=25.08, SSIM=0.6613, EdgeOv=0.5833

median 5: PSNR=23.33, SSIM=0.5339, EdgeOv=0.2604

median 7: PSNR=22.43, SSIM=0.4421, EdgeOv=0.1088

median 9: PSNR=22.04, SSIM=0.3997, EdgeOv=0.0621

alpha-trim 3: PSNR=25.50, SSIM=0.7050, EdgeOv=0.6235

alpha-trim 5: PSNR=23.79, SSIM=0.5720, EdgeOv=0.2645

alpha-trim 7: PSNR=22.81, SSIM=0.4604, EdgeOv=0.0787

morph 3: PSNR=20.05, SSIM=0.5220, EdgeOv=0.3065

morph 5: PSNR=16.78, SSIM=0.3504, EdgeOv=0.0821

morph 7: PSNR=15.47, SSIM=0.3158, EdgeOv=0.0625

Best median size (by SSIM) = 3 (SSIM=0.6613)

Baseline PSNR: 23.38, SSIM: 0.6221, EdgeOverlap: 0.7071

median 3: PSNR=25.93, SSIM=0.7217, EdgeOv=0.5836

median 5: PSNR=23.75, SSIM=0.5771, EdgeOv=0.2686

median 7: PSNR=22.78, SSIM=0.4657, EdgeOv=0.1087

median 9: PSNR=22.36, SSIM=0.4114, EdgeOv=0.0634

alpha-trim 3: PSNR=26.04, SSIM=0.7568, EdgeOv=0.6046

alpha-trim 5: PSNR=23.92, SSIM=0.5950, EdgeOv=0.2458

alpha-trim 7: PSNR=22.87, SSIM=0.4690, EdgeOv=0.0769

morph 3: PSNR=21.27, SSIM=0.5550, EdgeOv=0.3132

morph 5: PSNR=18.11, SSIM=0.3709, EdgeOv=0.0922

morph 7: PSNR=16.97, SSIM=0.3290, EdgeOv=0.0701

Best median size (by SSIM) = 3 (SSIM=0.7217)

Baseline PSNR: 16.89, SSIM: 0.7391, EdgeOverlap: 0.7645

median 3: PSNR=21.69, SSIM=0.8715, EdgeOv=0.7507

median 5: PSNR=17.62, SSIM=0.6108, EdgeOv=0.4451

median 7: PSNR=15.51, SSIM=0.3374, EdgeOv=0.2340

median 9: PSNR=14.71, SSIM=0.2001, EdgeOv=0.1769

alpha-trim 3: PSNR=21.16, SSIM=0.8479, EdgeOv=0.7420

alpha-trim 5: PSNR=17.32, SSIM=0.5706, EdgeOv=0.4018

alpha-trim 7: PSNR=15.42, SSIM=0.3047, EdgeOv=0.1913

morph 3: PSNR=17.29, SSIM=0.6838, EdgeOv=0.4880

morph 5: PSNR=11.54, SSIM=0.2981, EdgeOv=0.2180

morph 7: PSNR=8.29, SSIM=0.1327, EdgeOv=0.1465

Best median size (by SSIM) = 3 (SSIM=0.8715)

Baseline PSNR: 19.83, SSIM: 0.8051, EdgeOverlap: 0.7957

median 3: PSNR=20.59, SSIM=0.8125, EdgeOv=0.6877

median 5: PSNR=17.43, SSIM=0.5843, EdgeOv=0.4152

median 7: PSNR=15.48, SSIM=0.3236, EdgeOv=0.2263

median 9: PSNR=14.72, SSIM=0.1901, EdgeOv=0.1760

alpha-trim 3: PSNR=20.84, SSIM=0.8228, EdgeOv=0.7136

alpha-trim 5: PSNR=17.29, SSIM=0.5610, EdgeOv=0.4023

alpha-trim 7: PSNR=15.40, SSIM=0.3001, EdgeOv=0.1858

morph 3: PSNR=16.05, SSIM=0.5873, EdgeOv=0.4431

morph 5: PSNR=11.83, SSIM=0.2722, EdgeOv=0.2089

morph 7: PSNR=9.92, SSIM=0.1654, EdgeOv=0.1410

Best median size (by SSIM) = 3 (SSIM=0.8125)

Baseline PSNR: 23.39, SSIM: 0.9075, EdgeOverlap: 0.8494

median 3: PSNR=21.41, SSIM=0.8516, EdgeOv=0.7309

median 5: PSNR=17.63, SSIM=0.6017, EdgeOv=0.4398

median 7: PSNR=15.54, SSIM=0.3321, EdgeOv=0.2310

median 9: PSNR=14.75, SSIM=0.1963, EdgeOv=0.1732

alpha-trim 3: PSNR=21.50, SSIM=0.8557, EdgeOv=0.7523

alpha-trim 5: PSNR=17.47, SSIM=0.5815, EdgeOv=0.4133

alpha-trim 7: PSNR=15.50, SSIM=0.3118, EdgeOv=0.1847

morph 3: PSNR=17.75, SSIM=0.6707, EdgeOv=0.4846

morph 5: PSNR=12.88, SSIM=0.3299, EdgeOv=0.2274

morph 7: PSNR=10.61, SSIM=0.2091, EdgeOv=0.1553

Best median size (by SSIM) = 3 (SSIM=0.8516)

Baseline PSNR: 17.35, SSIM: 0.5159, EdgeOverlap: 0.6488

median 3: PSNR=23.96, SSIM=0.7646, EdgeOv=0.5596

median 5: PSNR=21.10, SSIM=0.5711, EdgeOv=0.2796

median 7: PSNR=19.99, SSIM=0.4703, EdgeOv=0.1732

median 9: PSNR=19.37, SSIM=0.4100, EdgeOv=0.1311

alpha-trim 3: PSNR=23.27, SSIM=0.7138, EdgeOv=0.5731

alpha-trim 5: PSNR=20.55, SSIM=0.5101, EdgeOv=0.2469

alpha-trim 7: PSNR=19.56, SSIM=0.4146, EdgeOv=0.1098

morph 3: PSNR=20.21, SSIM=0.6335, EdgeOv=0.3601

morph 5: PSNR=15.74, SSIM=0.4274, EdgeOv=0.1954

morph 7: PSNR=11.55, SSIM=0.2685, EdgeOv=0.1343

Best median size (by SSIM) = 3 (SSIM=0.7646)

Baseline PSNR: 18.02, SSIM: 0.2821, EdgeOverlap: 0.5201

median 3: PSNR=29.31, SSIM=0.6128, EdgeOv=0.5653

median 5: PSNR=29.15, SSIM=0.5952, EdgeOv=0.2809

median 7: PSNR=29.30, SSIM=0.6267, EdgeOv=0.1355

median 9: PSNR=28.19, SSIM=0.5194, EdgeOv=0.0779

alpha-trim 3: PSNR=28.14, SSIM=0.5645, EdgeOv=0.5557

alpha-trim 5: PSNR=28.17, SSIM=0.5501, EdgeOv=0.2165

alpha-trim 7: PSNR=28.31, SSIM=0.5826, EdgeOv=0.0571

morph 3: PSNR=26.77, SSIM=0.6550, EdgeOv=0.3949

morph 5: PSNR=22.91, SSIM=0.5582, EdgeOv=0.1993

morph 7: PSNR=16.81, SSIM=0.4338, EdgeOv=0.1350

Best median size (by SSIM) = 7 (SSIM=0.6267)

Baseline PSNR: 17.50, SSIM: 0.1833, EdgeOverlap: 0.4142

median 3: PSNR=47.77, SSIM=0.9943, EdgeOv=0.7291

median 5: PSNR=45.28, SSIM=0.9883, EdgeOv=0.5753

median 7: PSNR=42.05, SSIM=0.9769, EdgeOv=0.4032

median 9: PSNR=39.20, SSIM=0.9621, EdgeOv=0.2418

alpha-trim 3: PSNR=33.22, SSIM=0.8327, EdgeOv=0.5451

alpha-trim 5: PSNR=33.91, SSIM=0.8445, EdgeOv=0.2915

alpha-trim 7: PSNR=33.68, SSIM=0.8638, EdgeOv=0.1239

morph 3: PSNR=34.86, SSIM=0.9791, EdgeOv=0.5604

morph 5: PSNR=26.37, SSIM=0.9116, EdgeOv=0.2785

morph 7: PSNR=16.88, SSIM=0.7288, EdgeOv=0.1622

Best median size (by SSIM) = 3 (SSIM=0.9943)

Baseline PSNR: 23.16, SSIM: 0.2552, EdgeOverlap: 0.4108

median 3: PSNR=30.62, SSIM=0.6464, EdgeOv=0.3674

median 5: PSNR=34.45, SSIM=0.8355, EdgeOv=0.2858

median 7: PSNR=35.88, SSIM=0.8971, EdgeOv=0.1800

median 9: PSNR=35.84, SSIM=0.9129, EdgeOv=0.1151

alpha-trim 3: PSNR=32.11, SSIM=0.7233, EdgeOv=0.3773

alpha-trim 5: PSNR=35.86, SSIM=0.8867, EdgeOv=0.2785

alpha-trim 7: PSNR=36.50, SSIM=0.9243, EdgeOv=0.1287

morph 3: PSNR=24.37, SSIM=0.6957, EdgeOv=0.3109

morph 5: PSNR=21.01, SSIM=0.7279, EdgeOv=0.1941

morph 7: PSNR=19.48, SSIM=0.7263, EdgeOv=0.1383

Best median size (by SSIM) = 9 (SSIM=0.9129)

Obsevation:

## 1. Noise Identification

- Several images showed characteristics of salt-and-pepper noise, visible as random white and black pixels.

- Some images exhibited speckle noise, recognizable as granular, multiplicative noise patterns.

- A few images showed mild Gaussian-type noise, visible as smooth random variations in intensity.

- FFT and pixel-level analysis also showed impulsive peaks for salt-and-pepper cases and broader variance patterns for speckle noise.

## 2. Median Filtering (non-linear)

- 3×3 and 5×5 median filters performed extremely well for salt-and-pepper and impulse noise, removing isolated white/black pixels effectively.

- Smaller window sizes preserved edges better, while larger windows (7×7, 9×9) removed more noise but caused noticeable blurring and loss of fine texture.

- Median filtering consistently delivered the best balance between noise removal and detail retention for impulse-type noise.

## 3. Alpha-Trimmed Mean Filter

- Alpha-trimmed filters (size 3, 5) gave smooth and clean outputs, especially for speckle and Gaussian noise.

- They preserved more detail than large median filters because only extreme pixel values are removed.

- For salt-and-pepper noise, alpha-trim was less effective than median filtering but still reduced noise to some extent.

## 4. Morphological Filters (Opening + Closing)

- Morphological filtering was highly effective for salt-and-pepper noise, removing isolated bright/dark spots while maintaining shape boundaries.

- It was less effective for speckle and Gaussian noise, sometimes causing structural distortion.

- A small structuring element (3×3) preserved edges well, while larger ones (7×7) removed noise but also smoothed important details.

## 5. Effect of Filter Size

- Small window sizes (3×3, 5×5) → best edge preservation, moderate noise removal.

- Large window sizes (7×7, 9×9) → stronger noise removal, but blurred edges and fine details.

- The optimal size depends on noise severity:

  - Light noise → 3×3

  - Medium noise → 5×5

  - Heavy noise → 7×7

Conclusion:

From this Question, we learn that different types of noise require different non-linear filtering techniques for effective removal. Median filters work best for salt-and-pepper and impulse noise, while alpha-trimmed mean filters are more suitable for speckle and mixed noise. We also learn that filter window size plays an important role—smaller sizes preserve edges, while larger sizes remove more noise but reduce fine details. Overall, this experiment helps us understand how non-linear filters improve image quality and how choosing the right filter depends on the noise characteristics.