# Marwadi University

Faculty of Engineering and Technology

Department of Information and Communication Technology

B.Tech - SEM: 5
Digital Signal and Image Processing (01CT1513)
Combined Question Papers with Answers

# Mid-Sem Exam I (October 2024)

Total Marks: 30   |   Time: 75 Minutes

## Question 1

Explain the following terms and their uses:

Impulse response:

Analog versus Digital System:

System Function:

Linear phase:

Stable System:

Phase Delay and Group Delay:

Answer:

Impulse response: Output of system for δ(n). Used to characterize LTI systems.
Analog vs Digital: Analog is continuous-time, digital is discrete-time. Digital systems allow flexibility, storage, and easy processing.
System Function: Z-transform of impulse response, characterizes system completely.
Linear Phase: FIR filters can have linear phase to avoid phase distortion.
Stable System: BIBO stability, sum of |h(n)| finite.
Phase/Group Delay: Measures delay introduced at each frequency; important in communications.

## Question 2

a) Find the circular convolution using DFT and IDFT between {1,2,3,0} and {1,0,1,0}

Answer: Perform DFT of both sequences, multiply pointwise, take IDFT. Result = {4,2,4,2}

b) Find 8-point DFT of {1,1,1,0,1,0} and 2^n

Answer: Apply DFT definition. {1,1,1,0,1,0,0,0} → spectrum values; For 2^n, geometric sum gives DFT coefficients.

OR Find 8-point DIT FFT of {1,0,1,0,2,2,1,0} → computed using radix-2 FFT flow.

## Question 3

a) Design Analog Butterworth Filter with given specs and convert to Digital using impulse invariance.

Answer: Steps: 1) Determine order N from specifications. 2) Find cutoff ωc. 3) Analog transfer function H(s). 4) Apply impulse invariance (s = ln(z)/T). Final digital filter achieved.

b) Differentiate FIR and IIR: FIR finite memory, always stable, linear phase possible. IIR uses recursion, efficient, may be unstable.

OR Explain DSP processor architecture: Harvard architecture, MAC unit, pipeline, addressing modes, specialized registers.

# Mid-Sem Exam II (October 2023)

Total Marks: 30   |   Time: 75 Minutes

## Question 1

Given y(n)=2y(n-1)+3x(n)+2x(n-1). Find system function, impulse response, frequency, magnitude, phase responses, output for u(n) and δ(n).

Answer:

System Function H(z)= (3+2z^-1)/(1-2z^-1).
Impulse response: h(n)=3(2^n)u(n)+2(2^(n-1))u(n-1).
Frequency Response: H(e^jw)=... 
Magnitude=|H(e^jw)|, Phase=∠H(e^jw).
For input u(n): Use convolution, output grows as geometric sequence.
For δ(n): Output = h(n).

## Question 2

a) Draw Direct Form I and II of y(n)=x(n)+5x(n-1)+2y(n-1)+3y(n-2).

Answer: Direct Form I uses separate delay chains for input & output. Direct Form II uses single chain (canonic form).

b) Find DTFT of (i) (n+1)u(n) and (ii) e^(jkn)u(n).

Answer: (i) Sum_{n=0}∞ (n+1)e^{-jωn} = 1/(1-e^{-jω})^2. (ii) Geometric series → 1/(1-e^{-j(ω-k)}).

OR Circular convolution of {1,3,2,3} and {1,1,1,1} → {9,9,7,8}.

## Question 3

a) 8-point DIT FFT of {1,1,1,1,0,0,2,0}.

Answer: Compute using radix-2 butterfly → Spectrum values.

b) Differentiate FIR vs IIR (same as above).

OR 8-point DIF FFT of all-ones sequence → Result nonzero at k=0 only (value=8).

b) IIR filter design methods: Impulse invariance, bilinear transform, matched z-transform.

# Open Book ESE (December 2024)

Total Marks: 50   |   Time: 120 Minutes

## Question 1

If Discrete-time system is defined, find impulse, step, frequency, magnitude, phase responses.

Answer: Impulse response h(n) defines system. Step response = Σh(k). Frequency response H(e^jw)=Σh(n)e^{-jωn}. Magnitude=|H|, Phase=∠H.

## Question 2

Design Butterworth LPF, convert to digital using Direct Form I, then FIR using rectangular window.

Answer: Steps: Find filter order, cutoff frequency, analog H(s), convert via impulse invariance, realize direct form I. Then FIR via window method: h(n)=hd(n)w(n).

## Question 3

Given image matrix, draw histogram, perform equalization, image transformation, and negation.

Answer: Histogram: count pixel frequencies. Equalization: redistribute intensities. Transformation: s=3r-5. Negation: s= L-1-r.

## Question 4

Apply mean, mode, median filtering (3x3) to given image X(n1,n2) with mask h(n1,n2).

Answer: Convolve with filter kernel. Mean: average values. Median: middle value. Mode: most frequent pixel.

## Question 5

Perform morphological operations (dilation, erosion, opening, closing) on given binary images.

Answer: Dilation expands, erosion shrinks, opening = erosion+dilation (remove noise), closing = dilation+erosion (fill holes).