# Marwadi University
Digital Signal & Image Processing
Mathematical Question Bank with Solutions

Instructions:
1. Attempt all questions.
2. Show full step-by-step solutions.
3. Assume suitable values wherever required.

## Question 1: Sampling and Aliasing

Given continuous-time signal:
x(t) = 2cos(2π·300t) + 0.5cos(2π·1200t)
(a) Sample at Fs = 4000 Hz and compute x[n] for n=0 to 7
(b) Sample at Fs = 2000 Hz, compute x[n], and show aliasing of 1200 Hz component.

### Solution:

Step 1: Formula: x[n] = 2cos(2π·300·n/Fs) + 0.5cos(2π·1200·n/Fs)
Step 2: For Fs=4000 Hz → ω1=0.15π, ω2=0.6π
x[n] = 2cos(0.15πn) + 0.5cos(0.6πn)
Values for n=0..7: [2.5,1.6275,0.7711,0.7174,-0.4635,-1.9142,-1.7476,-1.5709]
Step 3: For Fs=2000 Hz → Aliased freq = |1200-2000|=800 Hz
ω1=0.3π, ω2=0.8π
Values for n=0..7: [2.5,0.7711,-0.4635,-1.7476,-2.0225,0.5,1.2135,2.0566]

## Question 2: Image Representation and Resolution

(a) Compute memory required for grayscale 480x640 8-bit image
(b) Compute quantization noise variance for Δ=1

### Solution:

Step 1: Pixels = 480×640 = 307200
Step 2: 1 byte/pixel → 307200 bytes = 300 KB
Step 3: Variance = Δ²/12 = 1/12 ≈ 0.0833

## Question 3: Zooming and Shrinking

Image I = [[10,20,30],[40,50,60],[70,80,90]]
(a) Shrink by factor 2
(b) Upsample by factor 2 (NN and Bilinear)

### Solution:

Step 1: Shrink → [[10,30],[70,90]]
Step 2: Nearest Neighbor (6x6):
[[10,10,20,20,30,30],...]
Step 3: Bilinear interpolation examples: (1,1)=30, (2,3)=55

## Question 4: 1D Convolution and Correlation

x[n]=[1,2,3], h[n]=[1,0,-1]
(a) Linear conv
(b) Circular conv (N=4)
(c) Cross & Auto correlation

### Solution:

Linear conv: [1,2,2,-2,-3]
Circular conv N=4: [-2,2,2,-2]
Cross-corr r_xh[k]=-2..2: [3,2,-2,-2,-1]
Auto-corr r_xx[k]=-2..2: [3,8,14,8,3]

## Question 5: 2D Convolution and Filters

Image X=4x4, Kernel h=3x3
Compute Y(0,0), Y(1,1); mean, median, mode for (1,1)

### Solution:

Y(0,0)=50, Y(1,1)=1
Neighborhood=[6,2,3,9,2,6,8,6,3]
Mean=5, Median=6, Mode=6

## Question 6: Histogram Equalization

Image I=[[0,1,2],[2,3,4],[4,5,7]], L=8

### Solution:

Histogram h=[1,1,2,1,2,1,0,1]
CDF=[0.111,0.222,0.444,0.556,0.778,0.889,0.889,1.0]
Mapping s=[0,1,3,3,5,6,6,7]
Equalized image=[[0,1,3],[3,3,5],[5,6,7]]

## Question 7: Discrete Fourier Transform

x[n]=[1,0,1,0], N=4 → Compute DFT

### Solution:

X[k]=1+e^{-jπk}
X[0]=2, X[1]=0, X[2]=2, X[3]=0

## Question 8: Smoothing & Sharpening

Patch P=[[10,20,30],[40,50,60],[70,80,90]]
Mean filter & Laplacian

### Solution:

Mean = 50, Laplacian center = 0