# Mathematical Formulas: Relationship Between Pixels

# 1. Basic Definitions

A pixel at location (x, y) is denoted by: p(x, y).

The neighborhood of a pixel depends on connectivity type.

# 2. Types of Neighborhood

1. 4-neighbors (N4): N4(p) = {(x+1,y), (x-1,y), (x,y+1), (x,y-1)}

2. Diagonal neighbors (ND): ND(p) = {(x+1,y+1), (x+1,y-1), (x-1,y+1), (x-1,y-1)}

3. 8-neighbors (N8): N8(p) = N4(p) ∪ ND(p)

# 3. Connectivity

Defines whether pixels are connected (part of same region).

• 4-connectivity: pixels are connected if they share an edge.

• 8-connectivity: pixels are connected if they share edge or corner.

• m-connectivity: combination of 4 and diagonal connections, avoids ambiguity.

# 4. Adjacency

Two pixels p and q are adjacent if:

• They are neighbors (depending on 4, 8, or m-connectivity).

• Their gray-level values satisfy a condition (e.g., both belong to same set V).

Mathematically:

Adj(p,q) = 1, if q ∈ N(p) and f(p), f(q) ∈ V; else 0

# 5. Path & Connectivity

A path from p to q: p = p0, p1, p2, …, pn = q such that pi is adjacent to pi-1.

If such a path exists → p and q are connected.

# 6. Region & Boundary

Region (R): a set of pixels with similar properties.

Boundary (∂R): pixels in region R that have at least one neighbor outside R.

Mathematically: ∂R = { p ∈ R | N(p) ⊄ R }

# 7. Distance Measures

1. Euclidean Distance (ED): DE(p,q) = sqrt((x1 - x2)^2 + (y1 - y2)^2)

2. City-block Distance (D4): D4(p,q) = |x1 - x2| + |y1 - y2|

3. Chessboard Distance (D8): D8(p,q) = max(|x1 - x2|, |y1 - y2|)

# 8. Exam Question Patterns

1. Write formulas for 4, 8, diagonal neighbors.

2. Define adjacency & connectivity mathematically.

3. Find distance between two pixels using Euclidean, City-block, and Chessboard metrics.

4. Define region and boundary mathematically.

5. Given pixel coordinates, compute neighborhood and distance.