# Mathematical Formulas & Question Types
Spatial & Gray-Level Resolution

# 1. Spatial Resolution

Represents the level of detail (fineness of image representation).

Formulas:

Δx = Image Width (W) / Number of Columns (M)

Δy = Image Height (H) / Number of Rows (N)

P = M × N   (Total number of pixels)

Rs = Δx × Δy   (Area per pixel)

Question Types:

1. Find pixel size given image size and pixel count.

2. Find number of pixels in a given resolution.

3. Compare two digitizations and decide which has better resolution.

# 2. Gray-Level Resolution

Represents the number of intensity levels that each pixel can take.

Formulas:

L = 2^k   (Number of gray levels, k = bits per pixel)

Range = 0 to (L-1)

ΔI = (Max Intensity - Min Intensity) / L   (Quantization step size)

k = log2(L)   (Bits required for given gray levels)

Question Types:

1. Find number of gray levels for given bits.

2. Find bits required for a given number of gray levels.

3. Find quantization step size for given intensity range & bits.

# 3. Storage Requirement

Formulas:

B = (M × N) × k   (Total bits required)

S = B / 8   (Total bytes required)

1 KB = 1024 bytes, 1 MB = 1024 KB

Question Types:

1. Compute storage size of an image with given resolution & bits.

2. Find how many images can fit in given memory.

3. Compare memory needed for two images.

# 4. Combined Questions

Example Patterns:

1. Image digitization: Find pixel size, gray levels, and storage for given dimensions & bits.

2. Resolution change effect: Compare pixels when resolution increases.

3. Bit-depth change effect: Calculate % reduction in storage.

Reduction % = ((Old bits - New bits) / Old bits) × 100

# 5. Extra Useful Relations

L = 2^k   (Number of gray levels per pixel)

DR = 20 log10(L-1)   (Dynamic range in dB)

CR = Uncompressed Size / Compressed Size   (Compression ratio)

Bit-plane representation: Each image can be decomposed into k binary images (bit-planes).

# Mathematical Formulas: Zooming & Shrinking in Images

# 1. Image Scaling Basics

Scaling (zoom/shrink) is done by multiplying coordinates by a scaling factor.

(x', y') = (sx * x, sy * y)

If sx, sy > 1 → Zooming (magnification)

If 0 < sx, sy < 1 → Shrinking (reduction)

# 2. Zooming (Enlargement)

New image size: M' = M * sy , N' = N * sx

Nearest Neighbor (Pixel Replication): Each pixel is copied sx × sy times.

Bilinear Interpolation Formula:

f(x,y) ≈ (1-α)(1-β)f(i,j) + α(1-β)f(i+1,j) + (1-α)β f(i,j+1) + αβ f(i+1,j+1)

where α = x - i, β = y - j

# 3. Shrinking (Reduction)

New image size: M' = M / sy , N' = N / sx

Nearest Neighbor (Subsampling): Take every sx-th and sy-th pixel.

Averaging Method:

f'(i,j) = (1 / (sx * sy)) Σ Σ f(sy*i + m , sx*j + n)

where summation m=0..sx-1 , n=0..sy-1

# 4. General Scaling Matrix (Geometric Transformation)

[x']   [sx  0   0] [x]

[y'] = [0   sy  0] [y]

[1 ]   [0   0   1] [1]

# 5. Storage & Resolution Effect

Zooming: Increases image size (more pixels) but may create redundancy.

Shrinking: Decreases image size (fewer pixels), may cause information loss.

# 6. Exam Question Patterns

1. Compute new size of an image after zooming/shrinking.

2. Apply nearest neighbor / bilinear interpolation formula for a given pixel.

3. Find new storage requirement after scaling.

4. Compare quality between replication, bilinear, and averaging methods.

# Mathematical Formulas: Relationship Between Pixels

# 1. Basic Definitions

A pixel at location (x, y) is denoted by: p(x, y).

The neighborhood of a pixel depends on connectivity type.

# 2. Types of Neighborhood

1. 4-neighbors (N4): N4(p) = {(x+1,y), (x-1,y), (x,y+1), (x,y-1)}

2. Diagonal neighbors (ND): ND(p) = {(x+1,y+1), (x+1,y-1), (x-1,y+1), (x-1,y-1)}

3. 8-neighbors (N8): N8(p) = N4(p) ∪ ND(p)

# 3. Connectivity

Defines whether pixels are connected (part of same region).

• 4-connectivity: pixels are connected if they share an edge.

• 8-connectivity: pixels are connected if they share edge or corner.

• m-connectivity: combination of 4 and diagonal connections, avoids ambiguity.

# 4. Adjacency

Two pixels p and q are adjacent if:

• They are neighbors (depending on 4, 8, or m-connectivity).

• Their gray-level values satisfy a condition (e.g., both belong to same set V).

Mathematically:

Adj(p,q) = 1, if q ∈ N(p) and f(p), f(q) ∈ V; else 0

# 5. Path & Connectivity

A path from p to q: p = p0, p1, p2, …, pn = q such that pi is adjacent to pi-1.

If such a path exists → p and q are connected.

# 6. Region & Boundary

Region (R): a set of pixels with similar properties.

Boundary (∂R): pixels in region R that have at least one neighbor outside R.

Mathematically: ∂R = { p ∈ R | N(p) ⊄ R }

# 7. Distance Measures

1. Euclidean Distance (ED): DE(p,q) = sqrt((x1 - x2)^2 + (y1 - y2)^2)

2. City-block Distance (D4): D4(p,q) = |x1 - x2| + |y1 - y2|

3. Chessboard Distance (D8): D8(p,q) = max(|x1 - x2|, |y1 - y2|)

# 8. Exam Question Patterns

1. Write formulas for 4, 8, diagonal neighbors.

2. Define adjacency & connectivity mathematically.

3. Find distance between two pixels using Euclidean, City-block, and Chessboard metrics.

4. Define region and boundary mathematically.

5. Given pixel coordinates, compute neighborhood and distance.